10/2004

SIERRA ROOM TM STANDARD SPECIFICATIONS SHEET

Part 1 PRODUCT

Insulated Glass Enclosure to be Sierra Room Model No.____________, by Florian Solar Products, L.L.C., Georgetown, South Carolina (800) FLORIAN  No other greenhouse system substitutes shall be allowed without written request for prior approval 10 days BEFORE the bid opening date.  Any bid from a non-pre-approved greenhouse manufacturer may not be used or accepted by any bidding General Contractor.

1.  Framework to be standard thermally-broken, extruded aluminum frame, pre-cut and pre-drilled consisting of curved dome roof, or straight eave, front wall and two gable ends.

2.  Alloy and Temper of all framework members shall be of 6063-T5.  Structural support members shall be as recommended for strength, corrosion resistance and application of required finish;  ASTM B 221 for extrusions; ASTM B 209  for  sheet/Plate.  Strength of unit is to withstand a positive and negative wind loading of _______ pounds per square foot and a simulated (on exterior) snow loading of______ pounds per square foot.  Maximum deflection is not to exceed__________of the total unit span.

3.  Glazing shall be: Hermetically sealed, factory insulated, 1/8" tempered-over- 1/8" tempered, dual sealed, 7/8" O.A. thick at all flat glass and curved glass applications, made with a molecular sieve desiccant, primary seal of polyisobutylene and a secondary seal of polyurethane.  All glass to comply with the Insulated Glass Certification Council with ASTM E 773-83 and E 774-84a class CBA requirements (IGCC No. IGCC-514CBA)  Option: 1. SunClean self-cleaning glass by PPG to be used on the outboard pane. 2.  Laminated glass may be required by code for the inboard lite of the I.G. unit for overhead areas.  (Choose roof & wall performance below):

Good:	Clear tempered over clear tempered safety glass, dual sealed

Solar Heat Gain Co-efficient: no higher then .70

Outdoor visible reflectance: no higher then 15%

Visible light Transmittance: no higher then 79%

Better:	Recommended for the vertical WALL area

MC Low-E clear tempered over clear tempered safety glass, dual sealed.

Solar Heat Gain Co-efficient: no higher then  .37;

Outdoor visible reflectance: no higher then 12%

Visible light Transmittance: no higher then 69%

Best:	Recommended for the ROOF area

Super MC Low-E clear tempered over clear tempered safety glass,

Solar Heat Gain Co-efficient: no higher then  .23;

Outdoor visible reflectance: no higher then 32%

Visible light Transmittance: no higher then 47%

Triple:	MC Low-E clear tempered  /  MC Low-E  /  clear tempered, dual sealed.

Solar Heat Gain Co-efficient: no higher then  .33;

Outdoor visible reflectance: no higher then 15%

Visible light Transmittance: no higher then 56%

4.  Glazing Gaskets shall be E.P.D.M. (Ethylene Propylene Diene Polymer).

5.  Finish for all aluminum framing members and trim shall be manufacturer’s standard high solids baked-on polycron enamel finish; either bronze or white color standard.  Custom finishes available include:  Dark bronze hard coat /electrochemical anodized finish, Clear hard coat anodized finish, and powder coat finishes.

6.  Fasteners shall be aluminum, non-magnetic stainless steel, or other materials warranted by the manufacturer to be non-corrosive and compatible with aluminum components.

7.  Bay Spacing to be 2' - 6 7/8" o.c. ( 30 7/8" ), standard.

8.  Glazing Bars are to be tubular extrusions.  The vertical and horizontal extrusions shall have internal and external weep/condensation channels to direct moisture, which could collect on the interior of the glass, to the exterior of the solarium.  The sill member shall be sloped and weep channeled to the exterior to force moisture, which may be collected, to exit to the exterior.  The moisture shall weep/exit through  small hinged weep hole covers located periodically along the length of the sill member at the center of each bay. Interior and exterior sill trim covers will be included (Finish to be the same as the solarium aluminum).  All fasteners are located within the structure of the unit and not visible to the exterior or interior of the unit.  The Sierra Room  solarium shall incorporate a screw-less, tamper-proof, commercial type  nylon thermal glazing clip assembly system to secure glass units and outside glazing caps  without a "metal-to-metal" contact. The solarium shall incorporate screwless flat exterior cap on horizontal roof bars and on horizontal wall/gable bars to allow flow-over of exterior rain-water on the upward side of the cap.  Head  members and sill members shall have a non-conductive poured thermal break incorporated to prevent "metal-to-metal" contact.

Part 2 OPTIONS

1.  Access Doors shall be manufacturer's standard 60" wide x 80 3/4" high thermally-broken aluminum sliding door and frame, fully weather stripped with insulated tempered glass, privacy lockset, and sliding insect screen.

2.  Access doors shall be:  36"  (39 3/4" x 86" R.O.) commercial quality single swing door or 72" (75 3/4" x 86" R.O.) commercial quality double swing door;  1" o.a. glazing.   Panic hardware and handicap hardware available.   Notes:  1.  Various other doors can be custom substituted.  2.  Various custom rough openings can be supplied to meet specific needs.

3.  Ventilation Windows shall be manufacture's standard lockable  13 3/4" single lite (AW1); 31" single lite (AW2);  or 45 3/4" high triple lite (AW3) awning windows, glazed to match solarium unit. Insect screens included.

Part 3 EXECUTION

INSPECTION:

1.  The installer must examine the substratum and the conditions under which the work is to be installed and notify the contractor, in writing of any conditions detrimental, to the proper and timely completion of the work.  Do not proceed with the work until unsatisfactory conditions have been corrected in the manner acceptable to the installer. Once materials are delivered, materials become property of the owner. Vandalism, theft, and damage is not the responsibility of Florian Solar Products, L.L.C.

INSTALLATION:

1.  Set all items in their correct location as shown in the detail level, square, plumb, at proper elevations and in alignment with other work in accordance with the manufacturer's installation instructions and approved shop drawings.

2.  All joints between framing and the building structure shall be flashed and sealed in order to secure a water tight installation.

3.  Fasten all frame materials in place using backing masonry plugs or anchor straps as required.

4.  Upon completion of the installation, it shall be the contractor's responsibility to make all necessary final adjustments to attain normal operation of each door, vent, accessory and their mechanical hardware.